## Workflow HW 1 – DAAP

Step 1: frame the signals

Since we are working with non-stationary signals, we need to split the signals into short frames where the behavior is approximately stationary. To do such thing we split the signals in many frames by using specific window length, hop size and windowing type. The following steps will be done for each frame and then the result will be reconstructed by OLA.

Step 2: LPC analysis of speech

We approximate the frame as a linear combination of previous samples. The number of past samples is p. The difference between the original signal and the approximation is called prediction error and must be minimized to obtain the best coefficients of the linear combination (Wiener filtering problem). Thanks to those coefficients the shaping filter H_s(z) can be found, it represents the filter that shapes the residual into the original signal, is our vocal tract filter. Such filter contains the spectral envelope needed to reconstruct the original signal by staring from the residual (excitation, glottal pulse) signal. Remember that:

- Residual contains pitch and timing

- Shaping filter contains the timbre (spectral envelope) and the formants (they are the "loudest" frequencies produced in the vocal tract due to resonance, which filter the sound from the vocal cords)

Step 3: LPC analysis of music

In this case we do the same LPC analysis but instead of saving the shaping filter we save the excitation signal e_m(n). Also in this case the parameter is the number of past samples is p.

Step 4: combination

Now, by combining the shaping filter of the speech (vocal timber) and the excitation residual of the music (time and pitch) we obtain the talk-box effect. Observe that a gain (energy of the residual) must also be applied to the result in case is needed.

Step 5: reconstruction with OLA

Now, to combine all frames, decide an overlap and a window to use and use OLA method. Also remember to normalize.

Wiener Filters

To find the best LPC we use the wiener filter approach. To find the coefficients we want to minimize the MSE E(e^2). Indeed we search for the coefficients. Such that the derivative is null. Once obtained the Wiener-Hopf eq. the Levinson-Durbin recursion algorithm can be used to find solution.

Notes

Implementation possibility: Voiced / unvoiced detection?

Idea for other excitation type: white noise, train of pulses, …